# vid2rgb121 — Video Converter for the PicoMite

vid2rgb121.py converts an ordinary video file (mp4, mkv, avi, gif, mov…) into

frames a PicoMite HDMI/USB (or buffered‑TFT) can play, plus an optional

soundtrack. A small companion BASIC player reads each frame straight into the

framebuffer — no per‑pixel BASIC, no decoding in the interpreter.

It can emit three formats; pick one with a flag:

| Format | Flag | Bits/px | Display | Compression | Player | Firmware decode |
| --- | --- | --- | --- | --- | --- | --- |
| RGB121 | (default) | 4 | MODE 2 (320×240, 16 col) | RLE (4‑bit run) | playrgb121.bas | BLIT MEMORY (built‑in) |
| RGB332 raw | --rgb332 | 8 | MODE 5 (320×240, 256 col) | none | playrgb332.bas | direct MEMORY INPUT |
| RGB332 RLE | --rgb332-rle | 8 | MODE 5 (320×240, 256 col) | count+value (~3×) | playrgb332rle.bas | BLIT MEMORY332 |

All three carry the same audio sidecar (name.aud) and the same per‑frame

length‑prefixed envelope, so the players share one fast read loop.

# 1. Requirements

Python 3.8+ (tested on 3.14).

FFmpeg — used to decode/rescale video and extract audio.

⚠️ The PyPI package literally named ffmpeg (pip install ffmpeg, ~5 kB)

is not FFmpeg. It is a thin wrapper that calls an ffmpeg binary you must

already have. Installing it does nothing useful here.

# 2. Installing FFmpeg

## Option A — bundled FFmpeg via pip (recommended, no system changes)

pip install imageio-ffmpeg

This downloads a self‑contained static FFmpeg into your Python environment.

The converter finds it automatically. Remove later with pip uninstall imageio-ffmpeg.

## Option B — system‑wide FFmpeg

Put ffmpeg on your PATH:

Windows: winget install Gyan.FFmpeg (then open a new terminal)

macOS: brew install ffmpeg

Linux: sudo apt install ffmpeg

The converter prefers a PATH FFmpeg and falls back to the imageio-ffmpeg

binary. Check with ffmpeg -version.

# 3. Basic usage

python vid2rgb121.py INPUT OUTPUT [options]

INPUT — any video FFmpeg can read.

OUTPUT — the frame file to create. Audio is written beside it with the same

base name and a .aud extension (myclip.vc → myclip.aud).

Examples:

python vid2rgb121.py myclip.mp4 myclip.vc                 # RGB121 (default)
python vid2rgb121.py myclip.mp4 myclip.r332  --rgb332     # raw RGB332
python vid2rgb121.py myclip.mp4 myclip.r332c --rgb332-rle # compressed RGB332

At the end the converter prints the exact player settings (vidFile,

audFile, totalFrames, frame pacing).

💡 Pick a unique OUTPUT name. The audio step writes OUTPUT‑with‑.aud,

overwriting any existing file of that name.

# 4. Options

| Option | Default | Meaning |
| --- | --- | --- |
| --w N | 320 | Output width. Must match the player's framebuffer. |
| --h N | 240 | Output height. Must match the player's framebuffer. |
| --fps F | 22 | Output frame rate. The player must be paced to the same value. |
| --rgb332 | off | Raw RGB332, 1 byte/pixel, no header (MODE 5). |
| --rgb332-rle | off | Count+value RLE RGB332 (MODE 5); needs the BLIT MEMORY332 decoder. |
| --arate Hz | 22050 | Audio sample rate. Must be ≤ 44100 × (CPUspeed/126000); 22050 is safe at any CPU speed. |
| --stereo | off | Keep 2 audio channels (default downmixes to mono). |
| --no-audio | off | Skip audio extraction. |

--rgb332 and --rgb332-rle are mutually exclusive; omit both for RGB121.

# 5. Which format to choose

RGB121 (default) — 16 colours, but each frame is RLE‑compressed in the

format BLIT MEMORY already decodes, so it plays at full frame rate with no

new firmware. Best when 16 colours are enough.

RGB332 raw (--rgb332) — full 256‑colour, simplest playback (a single

MEMORY INPUT straight onto the screen), but uncompressed: 76 800 bytes

per 320×240 frame. SD read throughput (~0.95 MB/s on a typical card) caps this

at ~12 fps — see §7.

RGB332 RLE (--rgb332-rle) — 256‑colour like raw, but count+value

RLE shrinks frames ~3× (measured on both flat and photographic clips), lifting

the sustainable rate to ~38 fps. Requires the BLIT MEMORY332 firmware

command. This is the best 256‑colour option.

# 6. Playing on the PicoMite

Copy the output file (and .aud, if present) to the SD card.

Set the display base resolution once (persists). For 320×240 on HDMI/USB the

base must be 640×480, halved by the player's MODE:

OPTION RESOLUTION 640x480,315000

The ,315000 (315 MHz) only matters for tight audio sync; any speed works

without audio.

Edit the matching player and set the three values the converter printed:

DIM STRING vidFile = "myclip.vc"     ' or .r332 / .r332c
DIM STRING audFile = "myclip.aud"    ' set to "" if no audio
DIM FLOAT  fps     = 22              ' must equal the converter's --fps

RUN the player. ESC stops playback.

| Output | Player | Mode |
| --- | --- | --- |
| *.vc | playrgb121.bas | MODE 2 |
| *.r332 | playrgb332.bas | MODE 5 |
| *.r332c | playrgb332rle.bas | MODE 5 |

How the players stay fast. Each frame is pulled straight into a RAM buffer

with MEMORY INPUT (no INPUT$/LONGSTRING loop), then drawn directly to the

live screen — no FRAMEBUFFER CREATE, no FRAMEBUFFER COPY. RGB121 and

RGB332‑RLE decode through BLIT MEMORY / BLIT MEMORY332; raw RGB332 is copied

verbatim into MM.INFO(WRITEBUFF). Writing to the scanned‑out buffer can tear

slightly; that is the trade for skipping the copy. Each player paces frames to

its fps so the audio track stays locked.

# 7. Performance & throughput

Playback of the RGB332 formats is limited by SD read speed, not the Pico.

A 320×240 raw RGB332 frame is 76 800 bytes; at a typical ~0.95 MB/s that is only

about 12 fps. So:

Raw --rgb332: either accept ~12 fps, or encode at a matching rate

(--fps 10 … 12) so playback runs in real time and the audio stays in sync.

Encoding a 20–22 fps clip and playing it raw will run long (it can't keep up).

--rgb332-rle: ~3× smaller frames → ~38 fps sustainable, so the full

20–22 fps plays in real time. Prefer this over raw whenever the firmware has

BLIT MEMORY332.

RGB121: RLE‑compressed and 4 bits/pixel, so it is comfortably real‑time at

the default 22 fps.

Audio is always extracted at real‑time duration, so as long as the player is

paced to the encode --fps (and that fps is actually sustainable for the

format), A/V stays in sync.

# 8. Notes & tips

Aspect ratio. Frames are stretched to fill --w × --h; a non‑4:3 source

looks squashed. To letterbox instead (keep proportions, pad with black), change

the -vf filter in vid2rgb121.py to:

"-vf", f"fps={args.fps},scale={W}:{H}:force_original_aspect_ratio=decrease:flags=area,pad={W}:{H}:(ow-iw)/2:(oh-ih)/2",

Audio. Extracted as mono 8‑bit PCM WAV (PLAY WAV). Use --stereo to keep

two channels, --arate to change the rate, --no-audio to skip. If the source

has no audio track the converter says so and writes only the video; set

audFile = "" (or comment out PLAY WAV) in the player.

Resolution must match the mode. --w/--h are tied to the player's MODE.

The players ship for 320×240. To target another size, change --w/--h and the

player's MODE/buffer together.

# 9. Troubleshooting

| Symptom | Cause / fix |
| --- | --- |
| ffmpeg not found … | Install FFmpeg — pip install imageio-ffmpeg (§2A) or system‑wide. |
| Audio: skipped (source has no audio track) | Source has no audio. Normal; only the video is written. |
| Video looks stretched/squashed | Source isn't 4:3. Use the letterbox filter in §8. |
| Max %KHz sample rate on the Pico | --arate too high for the CPU speed. Use 22050, or raise OPTION CPUSPEED. |
| Raw RGB332 plays slower than recorded | SD can't sustain 76 KB/frame at that fps. Use --rgb332-rle, or lower --fps (§7). |
| Audio drifts out of sync | Player fps ≠ converter --fps, or the fps isn't sustainable for the format. |
| Only available with an RGB332 display | BLIT MEMORY332 was used outside MODE 5 / a non‑RGB332 panel. |
| Integer array too small (RGB121/RLE players) | A frame exceeded the player buffer. Enlarge DIM INTEGER buf%(...). |